AetherQ: A Multi-Agent AI System for Autonomous Superconducting Quantum Chip Research

Anonymous Authors

1 Quantum Technology Innovation Center, Yangtze River Delta, China

2 Institute of Quantum Computing, CETC, China

Abstract. The design, fabrication, and characterization of superconducting quantum chips involve deeply intertwined workflows spanning electromagnetic simulation, circuit quantization, process engineering, cryogenic measurement, and data analysis. Each stage demands specialized expertise and distinct software tools, creating significant barriers to efficient iteration. Here we introduce AetherQ, a multi-agent AI system that unifies the full lifecycle of superconducting quantum chip research behind a single natural-language interface. AetherQ orchestrates twelve specialized AI scientist agents—spanning chip design, electromagnetic simulation, theoretical physics, process engineering, device calibration, quantum error mitigation, and data analysis—over a shared tool execution layer that integrates industry-standard platforms including Qiskit Metal, KQCircuits, Ansys HFSS/Q3D, ARTIQ, CHIPMES, and Apache Doris. The system features (i) a closed-loop architecture where design intent flows from natural language through EDA tools to GDS layout and back through simulation and measurement feedback, (ii) an autonomous heartbeat engine that continuously monitors equipment, data quality, and experimental anomalies across four escalating intelligence tiers, and (iii) a theoretical physicist agent implementing nine functional modules (M0–M8) covering Hamiltonian modeling, noise budgeting, Bayesian parameter inversion, information-gain-driven experiment design, root-cause diagnosis, and design optimization. We demonstrate AetherQ on a 20-qubit tunable-coupler chip design task derived from a real fabrication specification, showing that the system can autonomously generate parameterized GDS layouts, execute Q3D capacitance extraction and EPR analysis, diagnose gate-error root causes, and propose next-iteration design modifications—all from conversational prompts.

Keywords: multi-agent system, superconducting quantum chip, autonomous research, LLM-based agent, chip design automation, closed-loop optimization.

# 1   Introduction

Superconducting quantum computing has advanced rapidly over the past decade, with processors scaling from single qubits to architectures exceeding one hundred qubits [1–5]. Google’s demonstration of quantum error correction below the surface code threshold [3] and IBM’s roadmap toward utility-scale quantum computing [4] highlight the accelerating pace of hardware development. In China, teams at the University of Science and Technology of China and the China Electronics Technology Group Corporation (CETC) have developed 66-qubit and 20-qubit processors respectively [5], underscoring the global scope of this effort.

Despite these advances, the research and development cycle for each chip generation remains remarkably labor-intensive. A single design iteration typically involves six distinct stages: (i) frequency planning and Hamiltonian parameter selection based on target gate fidelities and coherence requirements, (ii) layout design in specialized electronic design automation (EDA) tools such as Qiskit Metal [15] or KQCircuits [16], (iii) electromagnetic simulation using Ansys HFSS and Q3D Extractor for capacitance extraction and eigenmode analysis, (iv) cleanroom fabrication with multi-step lithography, thin-film deposition, and Josephson junction patterning, (v) cryogenic measurement at millikelvin temperatures including spectroscopy, T1/T2 characterization, randomized benchmarking, and gate set tomography, and (vi) data-driven diagnosis and parameter refinement to inform the next design cycle.

Each of these stages relies on distinct software ecosystems, data formats, and domain expertise. A chip designer must be proficient in electromagnetic theory, microwave engineering, quantum mechanics, cryogenic instrumentation, and statistical data analysis—a breadth of knowledge that is rarely mastered by a single researcher. Moreover, the handoff between stages is typically manual: a designer exports a GDS file, emails it to the fabrication team, waits for cooldown results, manually enters measurement data into spreadsheets, and iterates. This fragmented workflow not only slows iteration cycles but also introduces opportunities for human error, knowledge loss, and suboptimal design choices.

Large language models (LLMs) and LLM-based agent systems have recently demonstrated the potential to bridge such expertise gaps and automate complex scientific workflows. Through pre-training on vast corpora of scientific literature and code, modern LLMs acquire broad technical knowledge that can be applied through in-context learning [17,18]. When augmented with tool-use capabilities—the ability to invoke external software, databases, and APIs—LLMs can transition from passive knowledge retrieval to active task execution [19,20].

Motivated by these developments, we introduce AetherQ, a multi-agent AI system designed to unify the full lifecycle of superconducting quantum chip research behind a single natural-language interface. AetherQ orchestrates twelve specialized AI scientist agents over a shared tool execution layer, enabling closed-loop workflows that span design, simulation, fabrication tracking, measurement, analysis, and redesign. Our key contributions are:

(1) Full-lifecycle coverage. AetherQ integrates twelve agents across theory, design, simulation, fabrication, calibration, error mitigation, data analysis, and knowledge management, connected through 120+ tools to real-world platforms including Qiskit Metal, KQCircuits, Ansys HFSS/Q3D, ARTIQ, CHIPMES (MES with 401 database tables), and Apache Doris.

(2) Closed-loop architecture. A central orchestrator routes natural-language intent to domain-specific agents using keyword-based routing with 35+ domain keywords per agent. The ReAct-style tool-call loop enables agents to reason, act, observe, and iterate autonomously for up to 8 rounds per turn.

(3) Physics-first theoretical reasoning. The theoretical physicist agent implements nine functional modules (M0–M8) covering effective Hamiltonian construction via EPR quantization, open-system noise budgeting with T1/T2 decomposition, Bayesian parameter inversion with posterior uncertainty, information-gain-driven experiment design, DRAG/GRAPE pulse optimization, fault-tree root-cause diagnosis, multi-objective design optimization, and structured knowledge graph retrieval.

(4) Autonomous heartbeat engine. A four-tier background monitoring system (L0: 5-minute equipment checks; L1: 6-hour data quality audits; L2: 12-hour experiment suggestions; L3: 24-hour cross-domain insight mining) continuously surfaces actionable discoveries without human prompting.

(5) Real-world demonstration. We apply AetherQ to a 20-qubit tunable-coupler chip design derived from a CETC production specification, demonstrating end-to-end GDS generation, Q3D simulation, noise budget analysis, root-cause diagnosis, and design optimization—all from conversational prompts.

# 2   Related Work

## 2.1   LLM-Based Scientific Agents

The emergence of LLM-based agent systems has catalyzed a new paradigm for scientific automation. The AI Scientist [6] demonstrates end-to-end machine learning research including idea generation, implementation, and paper writing. PaperQA [7] enables retrieval-augmented question answering over full-text scientific papers. Kosmos [8] spans literature search, ideation, implementation, and report generation across biology and materials science. NovelSeek [21] explores hypothesis generation and validation. The Virtual Lab [22] demonstrates collaborative AI agents that design and experimentally validate scientific discoveries. ORGANA [23] structures complex research workflows by operating over formalized scientific artifacts.

In chemistry, ChemCrow [24] and Coscientist [25] demonstrated tool-using agents for chemical research, while ChemAgent [26] extends this through multi-agent coordination. MDCrow [27] automates molecular dynamics simulations. These systems share a common pattern: specialized agents equipped with domain tools, coordinated through an orchestration layer, and driven by natural-language interaction.

## 2.2   AI for Quantum Computing

AI-assisted quantum computing has gained traction across several fronts. El Agente Q [9] is an LLM-based multi-agent system that automates quantum chemistry workflows through hierarchical planning, achieving >87% task success on university-level exercises. Its cognitive architecture features working memory, long-term memory (procedural, semantic, episodic), and grounding mechanisms. El Agente Cuántico [10] extends this to quantum simulation, orchestrating expert agents across CUDA-Q, PennyLane, Qiskit, QuTiP, TeNPy, and Tequila to automate tasks including VQE, Lindblad dynamics, HEOM, quantum optimal control, tensor network methods, and quantum error correction.

In quantum hardware, k-agents [11] demonstrate autonomous calibration of superconducting qubits using reinforcement learning. QCR-LLM [28] integrates quantum algorithms for combinatorial problems into LLM reasoning. PhIDO [29] addresses automated design of integrated photonic circuits. Saggio et al. [30] report experimental reinforcement learning on a programmable nanophotonic platform.

## 2.3   Gap Analysis

Table 1 summarizes the positioning of AetherQ relative to existing systems. While El Agente Q/Cuántico excel at computational workflows and k-agents at hardware calibration, no existing system spans the full design–simulate–fabricate–measure–analyze–redesign loop. AetherQ fills this gap by integrating EDA, electromagnetic simulation, manufacturing execution, cryogenic measurement, and data analysis platforms under a single multi-agent framework.

Table 1. Comparison of AetherQ with existing AI agent systems for quantum research.

[Table 1: Rows = El Agente Q, El Agente Cuantico, k-agents, AetherQ; Columns = Domain, Scope, Hardware Integration, Full Lifecycle, # Agents, # Tools; AetherQ: SC chip R&D, Full lifecycle, Yes (12+ platforms), Yes, 12, 120+]

# 3   System Architecture

## 3.1   Layered Design

AetherQ adopts a six-layer architecture inspired by the OpenClaw agent framework [12], specialized for quantum chip research. Figure 1 shows the overall system architecture.

Fig. 1. AetherQ system architecture. The user interacts through a natural-language client (L0). The Gateway (L1) manages sessions and protocols. The Brain (L2) performs intent classification, MoE routing, and multi-model inference. The Agent layer (L3) hosts twelve specialized AI scientists. The Hands/Memory layer (L4) provides tool execution and knowledge persistence. The platform layer (L5) connects to six business platforms and a data middle platform.

The Gateway (L1) handles WebSocket and REST connections, session management, authentication, and streaming output. It routes requests to the Brain (L2), which performs intent classification using a keyword-based Mixture-of-Experts (MoE) router. Each agent is configured with 7–35 domain-specific keywords; compound tasks are routed to the Orchestrator for decomposition. The Brain supports multi-model inference with configurable fallback chains: it first attempts the user-configured LLM (e.g., OpenAI, DeepSeek, Ollama), and falls back to a built-in response generator if the LLM is unavailable, ensuring graceful degradation.

## 3.2   Twelve-Agent Team

AetherQ deploys twelve specialized agents ordered by the natural research workflow, as shown in Figure 2. This ordering reflects the typical chip development pipeline: theoretical analysis precedes design, design precedes simulation, simulation precedes fabrication, and so on.

Fig. 2. The twelve AI scientist agents in AetherQ, ordered by the research workflow: Orchestrator → Theoretical Physicist → Chip Designer → Simulation Engineer → Materials Scientist → Process Engineer → Device Operator → Measurement Scientist → Algorithm Engineer → Data Analyst → Knowledge Engineer → Project Manager.

Each agent is characterized by four attributes: (i) a system prompt defining its role, capabilities, and behavioral guidelines; (ii) a set of allowed tool prefixes controlling which Hands adapters it can invoke; (iii) a keyword list for intent routing; and (iv) a set of quick-action buttons exposed in the web UI for common tasks. For example, the Theoretical Physicist agent's system prompt spans ~500 words describing its nine-module capabilities, its tool prefixes include 'theorist_', 'knowledge_', and 'doris_query', and its keyword list contains 35 domain terms ranging from 'Hamiltonian' to 'Pareto'.

## 3.3   Tool Execution Layer (Hands)

The Hands layer registers 120+ tools across ten platform adapters, as shown in Table 2. Each adapter implements a dual-mode design: when the target platform is available, real API calls are executed; when unavailable, the adapter falls back to physics-informed mock responses. This ensures the system remains functional for reasoning and planning even in environments where not all platforms are installed.

Table 2. Tool registry summary across platform adapters.

[Table 2: Qiskit Metal (8 tools), KQCircuits (6), Simulation (7), Theoretical Physics (10), ARTIQ (4), Qiskit Pulse (6), Mitiq (4), CHIPMES (8), Data Platform (15), QEDA Internal (6), Knowledge (5). Total: 120+]

## 3.4   Tool Call Loop and Pipeline

Following the ReAct paradigm [13], each agent operates in a think–act–observe loop, as illustrated in Figure 3. The Brain constructs a message sequence including the agent's system prompt, the full conversation history, and JSON-schema definitions of available tools. The LLM then generates either a natural-language response (terminating the loop) or a structured tool call specifying the tool name and arguments. If a tool call is issued, the Hands layer dispatches it to the corresponding adapter, executes it, and returns the result as a new message. This result is appended to the conversation history, and the LLM is invoked again. The loop continues for up to N_max = 8 rounds, enabling multi-step reasoning chains.

Fig. 3. The ReAct-style tool-call loop in AetherQ. The LLM alternates between reasoning (generating text or tool calls) and acting (executing tools through the Hands layer). Each tool execution result is fed back to the LLM for the next reasoning step. When tool calls occur during a chat session, a conversation pipeline is automatically created to record every step with full provenance.

When a tool call occurs during a chat session, the orchestrator automatically creates a conversation pipeline (CL-{uuid}) that records each step: the agent name, tool invoked, arguments, result summary, and timestamp. This pipeline is visible in the web UI and provides full provenance for reproducibility. For template-based design tasks (e.g., 20-qubit or 100-qubit chip design), dedicated pipeline runners execute pre-defined multi-stage workflows with gate checks, stage approvals, and report generation.

# 4   Theoretical Physicist Agent

The theoretical physicist agent is the physics reasoning core of AetherQ. Unlike general-purpose LLM reasoning, which relies on pattern matching over training data, this agent implements nine structured functional modules (M0–M8) grounded in superconducting circuit theory [14]. Figure 4 shows the module architecture and data flow.

Fig. 4. Architecture of the theoretical physicist agent showing nine functional modules (M0–M8) and their data exchange through standardized objects (DeviceGraph, HamiltonianModel, NoiseModel, CalibratedModelState, ExperimentPlan, DiagnosisReport, DesignProposal, KnowledgeEvidencePack).

## 4.1   Effective Hamiltonian Modeling (M1)

Module M1 constructs effective Hamiltonian models from the DeviceGraph using EPR quantization, black-box quantization, or lumped-element methods. For a transmon qubit with Josephson energy E_J and charging energy E_C, the transition frequency is approximated as f_01 ≈ (8 E_J E_C)^{1/2} - E_C, and the anharmonicity as α ≈ -E_C. The module extracts qubit frequencies, anharmonicities, coupling strengths g, dispersive shifts χ, ZZ interactions, and mode participation ratios for all qubits and couplers in the device. It also performs approximation validity checks: whether the two-level approximation holds (E_J/E_C > 20), whether the rotating-wave approximation is valid, and whether the dispersive regime condition |g| << |Δ| is satisfied.

## 4.2   Noise Budgeting (M2)

Module M2 decomposes T1 and T2 into constituent physical mechanisms. The T1 relaxation rate is modeled as a sum of independent channels: 1/T1 = 1/T1_Purcell + 1/T1_dielectric + 1/T1_TLS + 1/T1_QP + 1/T1_radiation, where each term corresponds to Purcell decay through the readout resonator, dielectric loss in the substrate and interfaces, two-level system (TLS) defects, quasiparticle tunneling, and radiative loss respectively. The dephasing rate is similarly decomposed: 1/T2 = 1/(2T1) + 1/Tφ_flux + 1/Tφ_thermal + 1/Tφ_charge, where the pure dephasing contributions arise from 1/f flux noise, thermal photon population in the readout resonator, and charge noise. The module outputs a ranked noise budget and a sensitivity matrix identifying the highest-leverage parameter changes.

## 4.3   Root-Cause Diagnosis (M6)

Module M6 implements fault-tree reasoning for performance anomalies. Given an observed symptom (e.g., CZ gate fidelity below target), the module constructs a tree of candidate root causes, assigns probabilistic confidence scores based on available evidence, identifies supporting and contradicting observations, and recommends the shortest verification experiment path. The diagnosis covers four anomaly categories: gate error, T1 degradation, frequency drift, and readout error, with 3–4 candidate mechanisms per category. Each candidate is annotated with references to the physics knowledge base (M8).

## 4.4   Design Optimization (M7)

Module M7 synthesizes the outputs of M2 (noise budget), M3 (calibrated parameters), and M6 (diagnosis) into actionable design proposals. It generates three tiers of recommendations: immediate parameter adjustments (pulse amplitudes, bias points, readout frequencies), mid-term structural changes (coupler parameters, airbridge density, Purcell filters), and next-iteration chip redesign (frequency plan, layout topology, packaging). Each proposal includes a risk assessment and a Pareto trade-off analysis over T1, gate speed, fabrication yield, and frequency crowding.

# 5   Chip Design Pipeline

## 5.1   20-Qubit Tunable-Coupler Specification

We demonstrate AetherQ's design capabilities on a 20-qubit chip following a production specification from CETC [5]: chip size 12.5 mm × 12.5 mm; one-dimensional chain topology with 19 tunable couplers; fixed-frequency Xmon qubits (Q_odd = 5.152 GHz, Q_even = 4.650 GHz); tunable coupler at 6.844 GHz with E_C = 348 MHz and Manhattan SQUID junction; 20 readout resonators spanning 7.0–7.97 GHz on 5 feedlines; 48 SMP connectors (20 XYZ controls, 19 coupler Z-lines, 5 input, 4 output); and CPW impedance Z_0 = 50 Ω (s = 10 μm, w = 5 μm on sapphire substrate). Table 3 summarizes the key design parameters.

Table 3. Design parameters for the 20-qubit tunable-coupler chip.

[Table 3: Qubit | f_01 (GHz) | f_r (GHz) | L_r (mm) | E_C (MHz) | E_J (GHz); Q1: 5.152, 7.378, 3.666, 260.35, 14.21; Q2: 4.650, 7.073, 3.835, 260.97, 11.68; ... Q19: 5.152, 7.966, 3.339, 260.35, 14.21; Q20: 4.650, 7.660, 3.479, 260.97, 11.68; All couplers: f_T = 6.844 GHz, E_C = 348 MHz, E_J = 18.58 GHz]

## 5.2   Design Workflow

The design pipeline, illustrated in Figure 5, proceeds as follows. (1) The user issues a natural-language prompt: 'Design a 20-qubit tunable-coupler chip per the specification.' (2) The orchestrator routes to the Chip Designer agent. (3) The agent invokes a sequence of Qiskit Metal tools: metal_create_design (12.5 mm × 12.5 mm, CPW s=10 μm, w=5 μm) → metal_add_transmon (×20, with connection_pads B0/B1/B2/B3) → metal_add_route (×19, RouteMeander with total_length 6–7 mm) → metal_export_gds. (4) If Qiskit Metal's GDS renderer encounters compatibility issues (e.g., pandas DataFrame.append deprecation), a fallback gdstk-based generator produces the layout with Xmon cross geometry, SQUID structures, readout feedlines, and 48 wirebond pads. (5) The Simulation Engineer performs Q3D extraction and HFSS eigenmode analysis. (6) The Theoretical Physicist validates the design through Hamiltonian modeling and noise budgeting.

Fig. 5. The 20-qubit chip design pipeline in AetherQ, from natural-language prompt through GDS generation, EM simulation, and theoretical validation. Each box represents a tool call with the responsible agent labeled.

## 5.3   GDS Layout

Figure 6 shows the generated GDS layout viewed in KLayout. The layout contains approximately 1,000 shapes across 8 layers: ground plane (layer 0), metal (layer 1), gap (layer 2), Josephson junction (layer 3), wirebond pad (layer 4), chip frame (layer 1/10), label (layer 10), and alignment mark (layer 11). The 20 Xmon qubits are arranged along a diagonal one-dimensional chain with 19 tunable couplers positioned on the upper side. Q2 and Q5 use traditional Manhattan single-junction structures for comparison, while all other qubits and couplers use Manhattan SQUID junctions as specified in the design document.

Fig. 6. Generated GDS layout of the 20-qubit tunable-coupler chip viewed in KLayout. The layout shows the diagonal qubit chain (Q1–Q20), tunable couplers (T1–T19), five readout feedlines, 48 wirebond pads distributed across four edges, and organized fanout routing.

# 6   Experiments and Results

We evaluate AetherQ across five experimental tasks that exercise different subsystems of the agent framework. All experiments use the 20-qubit tunable-coupler chip as the target design.

## 6.1   Experiment 1: Hamiltonian Model Construction

Task: Construct an effective Hamiltonian model for the 20-qubit chip from the DeviceGraph. The user prompt is: 'Build a Hamiltonian model for the 20-qubit chip and check for frequency collisions.' The orchestrator routes to the Theoretical Physicist, which invokes theorist_build_device_graph followed by theorist_build_hamiltonian (EPR method, truncation dimension 4). Table 4 compares agent-predicted parameters with design values.

Table 4. Hamiltonian model validation: agent predictions vs. design specification.

[Table 4: Parameter | Design Value | Agent Prediction | Agreement; ω_Q,odd: 5.152 GHz, 5.148±0.004 GHz, √; ω_Q,even: 4.650 GHz, 4.652±0.003 GHz, √; α_Q: -260 MHz, -258±2 MHz, √; g_coupling: ~15 MHz, 14.8±1.5 MHz, √; ζ_ZZ (off): <20 kHz, 18±5 kHz, √; Collision warnings: 0, 0, √]

The agent correctly identifies that no frequency collisions exist in the designed parameter space, and that all approximation conditions (two-level, RWA, dispersive) are satisfied with E_J/E_C ≈ 55 for Q_odd and ≈45 for Q_even.

## 6.2   Experiment 2: Noise Budget and Sensitivity Analysis

Task: Compute the noise budget for Q1 and identify the dominant decoherence mechanisms. The agent invokes theorist_noise_budget with the Hamiltonian model ID and measured T1 = 45 μs, T2 = 30 μs. Figure 7 shows the resulting T1 and T2 decomposition.

Fig. 7. Noise budget decomposition for Q1. Left: T1 breakdown showing dielectric loss as the dominant mechanism (contributing ~35% of the total relaxation rate), followed by TLS defects (~22%), Purcell decay (~14%), and quasiparticle tunneling (~5%). Right: Tφ breakdown showing 1/f flux noise as the dominant dephasing mechanism (~42%), followed by thermal photon dephasing (~13%).

The sensitivity matrix output identifies two key findings: (1) Reducing the dielectric loss tangent from 3×10^{-6} to 1.5×10^{-6} would improve T1 by approximately 20%. (2) Improving magnetic shielding to reduce the flux noise amplitude A_Φ by a factor of 2 would improve T2 by approximately 30%. These actionable insights are directly consumed by the design optimization module (M7).

## 6.3   Experiment 3: Root-Cause Diagnosis

Task: Diagnose why a simulated CZ gate between Q1 and Q2 achieves only 98.5% fidelity instead of the target 99%. The agent invokes theorist_diagnose with anomaly_type='gate_error_high'. Table 5 shows the diagnosis output.

Table 5. Root-cause diagnosis for CZ gate error.

[Table 5: Rank | Root Cause | Confidence | Supporting Evidence | Verification; 1: Frequency collision near CZ point, 0.45, Chevron shows avoided crossing + high leakage, High-resolution Chevron scan; 2: Flux bias drift, 0.30, Ramsey shows frequency drift + time-varying gate error, Long-term Ramsey tracking; 3: Package mode hybridization, 0.15, Broadband spectroscopy shows anomalous mode, Spectroscopy vs. temperature]

The diagnosis provides not only ranked hypotheses but also the shortest verification path: performing a high-resolution Chevron scan would either confirm or eliminate the top-ranked hypothesis with a single experiment, whereas exhaustive characterization would require multiple measurement campaigns. This information-theoretic approach to experimental planning is a key advantage of the physics-structured agent over unconstrained LLM reasoning.

## 6.4   Experiment 4: Experiment Design

Task: Plan an 8-hour measurement session to identify the dominant noise source for Q1. The agent invokes theorist_plan_experiment with objective='identify_dominant_noise' and budget_hours=8. The output schedules five experiments in priority order: (1) T1 vs. frequency (flux sweep), 60 min, to distinguish Purcell from dielectric loss; (2) T2 Ramsey vs. Echo comparison, 30 min, to quantify low-frequency noise; (3) CPMG noise spectroscopy, 90 min, to extract the noise power spectral density; (4) temperature-dependent T1, 120 min, to distinguish quasiparticle from thermal photon mechanisms; (5) residual excited-state population measurement, 20 min, to assess effective temperature. The total scheduled time is 5.3 hours, leaving margin for setup and contingencies. Adaptive stopping criteria are defined: if the top-ranked hypothesis reaches >85% confidence after any experiment, the remaining schedule is truncated.

## 6.5   Experiment 5: Full Chip Simulation

Task: Perform a complete simulation of the first 5 qubits (Q1–Q5) including Q3D capacitance extraction, LOM analysis, EPR analysis, and HFSS eigenmode simulation. The agent invokes sim_full_chip, which orchestrates four simulation stages per qubit. Table 6 summarizes the results for Q1–Q5.

Table 6. Full-chip simulation results for Q1–Q5.

[Table 6: Qubit | C_self (fF) | g_nearest (MHz) | f_01 (GHz) | E_C (MHz) | E_J (GHz) | p_junction | T1_dielectric (μs) | Q_factor; Q1: 62.3, 15.2, 5.15, 258, 14.1, 0.951, 85.3, 52000; Q2: 58.7, 16.1, 4.64, 262, 11.7, 0.948, 92.1, 48000; Q3: 63.1, 14.8, 5.16, 259, 14.3, 0.953, 83.7, 54000; Q4: 57.9, 15.9, 4.66, 261, 11.6, 0.946, 90.5, 47000; Q5: 64.2, 15.0, 5.14, 260, 14.2, 0.952, 86.2, 53000]

The simulation results show consistent agreement with the design specification across all five qubits. The junction participation ratios p_junction > 0.94 indicate that the transmon regime is well maintained, and the predicted T1 limits from dielectric loss (83–92 μs) are consistent with state-of-the-art superconducting qubit performance.

## 6.6   Experiment 6: Autonomous Heartbeat Discoveries

We evaluate the heartbeat engine over a simulated 24-hour monitoring period. Table 7 shows the discoveries generated across the four intelligence tiers.

Table 7. Autonomous discoveries from the heartbeat engine over 24 hours.

[Table 7: Tier | Interval | Discovery | Action Taken; L0 (5 min): Equipment alarm on LITHO-03, Alert sent to operator; L0 (5 min): 2 ETL pipelines stopped, Restart triggered; L1 (6 hours): Yield average 88% < 90% threshold, Root-cause analysis initiated; L1 (6 hours): calibration table quality score 85%, Data quality flag raised; L2 (12 hours): Q3 T1=32μs < 35μs threshold, Recalibration recommended; L3 (24 hours): High-yield batches correlate with cal fidelity 99.2% vs 97.8%, Design insight logged for next iteration]

The heartbeat engine produced six actionable discoveries without any human prompting. The L3 cross-domain insight—correlating fabrication yield with calibration fidelity—is particularly valuable as it would typically require a data analyst to manually join datasets from the MES system and the measurement database. This demonstrates the system's ability to surface non-obvious relationships across the chip lifecycle.

# 7   Discussion

## 7.1   Strengths

AetherQ demonstrates three key strengths. First, the physics-structured theoretical physicist agent provides reasoning that is both more reliable and more interpretable than unconstrained LLM generation. By decomposing complex physics problems into well-defined modules with standardized interfaces, the system can combine LLM flexibility with domain rigor. Second, the graceful degradation design ensures that the system remains useful even when not all platforms are available—a common situation in research environments where instruments and licenses may be shared. Third, the conversation pipeline mechanism provides full provenance for every agent action, enabling reproducibility and auditability that are essential in scientific research.

## 7.2   Limitations

Several limitations should be noted. (1) Without an Ansys HFSS desktop installation, electromagnetic simulations rely on analytical approximations (theory-mode) rather than full 3D field solutions. While the analytical results are physically informed, they cannot capture geometry-dependent effects such as parasitic modes or substrate radiation. (2) The GDS layout generator produces topologically correct layouts that match the design specification, but pixel-level reproduction of production masks requires additional manual refinement in professional EDA tools. (3) LLM function-calling occasionally produces malformed arguments or selects suboptimal tools, which is mitigated by validation layers and retry logic but not fully eliminated. (4) The current deployment is single-server, limiting concurrent agent execution to one user session at a time.

## 7.3   Roadmap

The development roadmap proceeds in three phases. Phase 1 (current) establishes the diagnostic closed loop: data ingestion → Hamiltonian modeling → calibration → diagnosis → next experiment recommendation. Phase 2 will add real instrument integration with ARTIQ hardware controllers and active learning for adaptive measurement scheduling. Phase 3 targets autonomous chip redesign with manufacturing-aware multi-objective optimization, closing the full design–fabricate–measure–redesign loop.

# 8   Conclusions

We have presented AetherQ, a multi-agent AI system for autonomous superconducting quantum chip research. The system integrates twelve specialized agents over a unified tool execution layer connecting to 120+ tools across Qiskit Metal, KQCircuits, Ansys HFSS/Q3D, ARTIQ, CHIPMES, Apache Doris, and other platforms. The theoretical physicist agent implements nine functional modules providing physics-grounded reasoning with explicit uncertainty quantification for Hamiltonian modeling, noise budgeting, parameter inversion, experiment design, pulse optimization, root-cause diagnosis, design optimization, and knowledge retrieval.

Applied to a production-derived 20-qubit tunable-coupler chip design, AetherQ demonstrates end-to-end capabilities: generating parameterized GDS layouts with Xmon qubits and SQUID couplers, executing Q3D capacitance extraction and EPR analysis, decomposing T1/T2 into constituent noise mechanisms, diagnosing CZ gate errors with probabilistic root-cause ranking, planning information-optimal measurement sequences, and generating three-tier design optimization proposals—all orchestrated through natural-language interaction.

The autonomous heartbeat engine further demonstrates that continuous, unsupervised monitoring can surface actionable cross-domain insights (e.g., yield–calibration correlations) that would otherwise require manual data integration. Together, these capabilities represent a step toward AI-native quantum hardware development, where the boundaries between design, simulation, fabrication, and characterization become increasingly fluid and where AI serves not merely as a tool but as an active participant in the scientific process.

Acknowledgments. This work was supported by [funding information to be added].

Disclosure of Interests. The authors have no competing interests to declare.

# References

1. Arute, F., et al.: Quantum supremacy using a programmable superconducting processor. Nature 574, 505–510 (2019)

2. Wu, Y., et al.: Strong quantum computational advantage using a superconducting quantum processor. Phys. Rev. Lett. 127, 180501 (2021)

3. Google Quantum AI: Quantum error correction below the surface code threshold. Nature 638, 920–926 (2025)

4. IBM Quantum: The IBM Quantum Development Roadmap. https://www.ibm.com/quantum/roadmap (2024)

5. CETC: 20-Qubit Tunable-Coupler Quantum Chip Design Specification. Internal Document TGQ-200-000-FA09-2025 (2025)

6. Lu, C., et al.: The AI Scientist: Towards Fully Automated Open-Ended Scientific Discovery. arXiv:2408.06292 (2024)

7. Lála, J., et al.: PaperQA: Retrieval-Augmented Generative Agent for Scientific Research. arXiv:2312.07559 (2023)

8. Li, X., et al.: Kosmos: An End-to-End AI Research Agent. arXiv (2024)

9. Zou, Y., et al.: El Agente: An Autonomous Agent for Quantum Chemistry. arXiv:2505.02484 (2025)

10. Gustin, I., et al.: El Agente Cuántico: Automating quantum simulations. arXiv:2512.18847 (2025)

11. Reuer, K., et al.: Autonomous calibration of superconducting qubits using k-agents. Nature (2025)

12. OpenClaw: Open-source agent framework. https://github.com/openclaw (2024)

13. Yao, S., et al.: ReAct: Synergizing Reasoning and Acting in Language Models. arXiv:2210.03629 (2022)

14. Internal Design Document: Theoretical Physicist Agent Skill Specification. AetherQ Project (2026)

15. Qiskit Metal: Open-source quantum hardware design. https://qiskit.org/metal (2024)

16. KQCircuits: IQM’s open-source superconducting chip design library. https://github.com/iqm-finland/KQCircuits (2024)

17. Brown, T., et al.: Language models are few-shot learners. NeurIPS 33, 1877–1901 (2020)

18. Wei, J., et al.: Chain-of-thought prompting elicits reasoning in large language models. NeurIPS 35 (2022)

19. Schick, T., et al.: Toolformer: Language models can teach themselves to use tools. NeurIPS 36 (2023)

20. Patil, S., et al.: Gorilla: Large language model connected with massive APIs. arXiv:2305.15334 (2023)

21. Wang, L., et al.: NovelSeek: When Agent Becomes the Scientist. arXiv (2025)

22. Swanson, K., et al.: The Virtual Lab: AI agents design new SARS-CoV-2 nanobodies. arXiv (2024)

23. Darvish Rouhani, B., et al.: ORGANA: A Robotic Assistant for Automated Chemistry Experimentation. arXiv (2024)

24. Bran, A., et al.: ChemCrow: Augmenting large-language models with chemistry tools. arXiv:2304.05376 (2023)

25. Boiko, D., et al.: Autonomous chemical research with large language models. Nature 624, 570–578 (2023)

26. Zhang, K., et al.: ChemAgent: Multi-Agent Coordination for Chemistry. arXiv (2025)

27. Schiltz, F., et al.: MDCrow: A multi-agent system for molecular dynamics. arXiv (2025)

28. Meng, F., et al.: QCR-LLM: Integrating quantum algorithms in LLM reasoning. arXiv (2025)

29. Xu, H., et al.: PhIDO: Automated design of integrated photonic circuits. arXiv (2024)

30. Saggio, V., et al.: Experimental quantum speed-up in reinforcement learning agents. Nature 591, 229–233 (2021)