AetherQ: A Multi-Agent AI System for Autonomous Superconducting Quantum Chip Research

Anonymous Authors

1 Quantum Technology Innovation Center, Yangtze River Delta, China

2 Institute of Quantum Computing, CETC, China

Abstract. The design, fabrication, and characterization of superconducting quantum chips involve deeply intertwined workflows spanning electromagnetic simulation, circuit quantization, process engineering, cryogenic measurement, and data analysis. Here we introduce AetherQ, a multi-agent AI system that unifies the full lifecycle of superconducting quantum chip research behind a single natural-language interface. AetherQ orchestrates twelve specialized AI scientist agents over a shared tool execution layer integrating Qiskit Metal, KQCircuits, Ansys HFSS/Q3D, ARTIQ, CHIPMES, and Apache Doris. The system features (i) a closed-loop architecture from natural language through EDA to GDS and back through simulation feedback, (ii) an autonomous heartbeat engine with four intelligence tiers, and (iii) a theoretical physicist agent with nine functional modules (M0–M8) covering Hamiltonian modeling, noise budgeting, Bayesian parameter inversion, experiment design, root-cause diagnosis, and design optimization. We demonstrate AetherQ on a 20-qubit tunable-coupler chip, showing end-to-end GDS generation, simulation, diagnosis, and redesign recommendation from conversational prompts.

Keywords: multi-agent system, superconducting quantum chip, autonomous research, LLM-based agent, chip design automation, closed-loop optimization.

# 1   Introduction

Superconducting quantum computing has advanced rapidly, with processors scaling from tens to over one hundred qubits [1–5]. Yet the R&D cycle remains labor-intensive. A single design iteration involves frequency planning, layout design in EDA tools, EM simulation, cleanroom fabrication, cryogenic measurement, and data-driven diagnosis—each relying on distinct software ecosystems and deep domain expertise rarely held by one researcher.

Large language models (LLMs) and agent systems have shown potential to bridge such gaps. El Agente Q [9] automates quantum chemistry workflows; El Agente Cuántico [10] extends this to quantum simulation; k-agents [11] demonstrate autonomous qubit calibration. However, none spans the full design–simulate–fabricate–measure–analyze–redesign loop. AetherQ fills this gap with five contributions: (1) full-lifecycle coverage with 12 agents and 120+ tools; (2) closed-loop tool-call orchestration; (3) a physics-structured theoretical physicist agent (M0–M8); (4) a four-tier autonomous heartbeat engine; (5) real-world demonstration on a 20-qubit tunable-coupler chip.

# 2   Related Work

## 2.1   LLM-Based Scientific Agents

The AI Scientist [6] demonstrates end-to-end ML research. PaperQA [7] enables retrieval-augmented QA over scientific papers. Kosmos [8] spans literature, ideation, and implementation. In chemistry, ChemCrow [24] and Coscientist [25] demonstrated tool-using agents, while ChemAgent [26] extends through multi-agent coordination.

## 2.2   AI for Quantum Computing

El Agente Q [9] achieves >87% task success on quantum chemistry exercises through hierarchical planning. El Agente Cuántico [10] orchestrates expert agents across CUDA-Q, PennyLane, Qiskit, QuTiP, TeNPy, and Tequila for VQE, Lindblad dynamics, HEOM, quantum control, tensor networks, and QEC. In hardware, k-agents [11] use RL for autonomous qubit calibration.

## 2.3   Gap Analysis

No existing system spans the full hardware lifecycle. Table 1 positions AetherQ relative to prior work.

Table 1. Comparison with existing AI agent systems for quantum research.

# 3   System Architecture

## 3.1   Layered Design

AetherQ adopts a six-layer architecture (Fig. 1): user layer (L0), Gateway (L1), Brain with MoE routing (L2), 12 AI scientist agents (L3), Hands/Memory tool+knowledge layer (L4), and platform layer (L5) connecting to Q-EDA, Ansys, CHIPMES, ARTIQ, Doris, and Grafana.

Fig. 1. AetherQ system architecture with six layers, Heartbeat engine, and Skills marketplace.

## 3.2   Twelve-Agent Team

The agents are ordered by research workflow (Fig. 2). Each agent has a system prompt, tool-prefix permissions, routing keywords, and quick-action buttons in the web UI.

Fig. 2. Twelve AI scientist agents ordered by the superconducting chip research workflow.

| System | Domain | Scope | HW Integration | Full Lifecycle | Agents | Tools |
| --- | --- | --- | --- | --- | --- | --- |
| El Agente Q [9] | Quantum Chem. | Computation | No | No | ~10 | ~20 |
| El Agente C. [10] | Quantum Sim. | Computation | Partial | No | 6 | ~30 |
| k-agents [11] | Qubit Calib. | Measurement | Yes | Partial | 1 | ~5 |
| AetherQ (ours) | SC Chip R&D | Full lifecycle | Yes (12+) | Yes | 12 | 120+ |

## 3.3   Tool Execution Layer

The Hands layer registers 120+ tools across ten platform adapters (Table 2). Each adapter implements dual-mode: real API when available, physics-informed mock when not.

Table 2. Tool registry across ten platform adapters (120+ total).

| Platform Adapter | Tools | Examples |
| --- | --- | --- |
| Qiskit Metal | 8 | create_design, add_transmon, export_gds |
| KQCircuits | 6 | create_chip, export_ansys, export_mask |
| Simulation | 7 | hfss_eigenmode, q3d_extraction, epr_analysis |
| Theoretical Physics | 10 | build_hamiltonian, noise_budget, diagnose |
| ARTIQ | 4 | run_pulse, run_scan, list_devices |
| Qiskit Pulse | 6 | full_calibration, cal_drag, cal_amplitude |
| Mitiq | 4 | zne, pec, dd, benchmark |
| CHIPMES (MES) | 8 | query_orders, db_schema (401 tables) |
| Data Platform | 15 | doris_query_sql, seatunnel_sync, qdata_text2sql |
| Knowledge/Library | 5 | search_knowledge, library_upload |

## 3.4   Tool Call Loop

Each agent operates in a ReAct think–act–observe loop (Fig. 3). The LLM generates text or a tool call; if a tool call, Hands executes and returns the result. This loops for up to 8 rounds. A conversation pipeline records every step for provenance.

Fig. 3. ReAct-style tool-call loop with up to 8 reasoning rounds per turn.

# 4   Theoretical Physicist Agent

The theoretical physicist implements nine modules (M0–M8) grounded in superconducting circuit theory (Fig. 4).

Fig. 4. Nine functional modules (M0–M8) of the theoretical physicist agent and their data flow through standardized objects.

## 4.1   Hamiltonian Modeling (M1)

M1 builds effective Hamiltonian models using EPR quantization. For a transmon with E_J and E_C: f_01 ≈ (8 E_J E_C)^{1/2} − E_C, anharmonicity α ≈ −E_C. Outputs include frequencies, coupling strengths g, dispersive shifts χ, ZZ interactions, and approximation validity checks.

## 4.2   Noise Budgeting (M2)

M2 decomposes 1/T1 = 1/T1_Purcell + 1/T1_dielectric + 1/T1_TLS + 1/T1_QP + 1/T1_radiation and 1/T2 = 1/(2T1) + 1/Tφ_flux + 1/Tφ_thermal + 1/Tφ_charge, generating ranked noise budgets and sensitivity matrices.

## 4.3   Root-Cause Diagnosis (M6) and Design Optimization (M7)

M6 implements fault-tree reasoning with probabilistic ranking. M7 synthesizes M2/M3/M6 outputs into three-tier design proposals (immediate/mid-term/next-iteration) with Pareto trade-off analysis.

# 5   Chip Design Pipeline

## 5.1   20-Qubit Specification

We demonstrate on a 20-qubit tunable-coupler chip [5]: 12.5 mm × 12.5 mm, 1D chain with 19 couplers, Xmon qubits (Q_odd=5.152 GHz, Q_even=4.650 GHz), coupler at 6.844 GHz, 48 SMP connectors, CPW Z_0=50 Ω (Table 3).

Table 3. Hamiltonian model parameters for the 20-qubit chip (selected qubits).

| Qubit | f_01 (GHz) | α (MHz) | E_J (GHz) | E_C (GHz) | E_J/E_C |
| --- | --- | --- | --- | --- | --- |
| Q1 | 5.174 | -260.0 | 13.78 | 0.2600 | 55 |
| Q2 | 5.215 | -260.0 | 14.17 | 0.2600 | 55 |
| Q3 | 5.236 | -260.0 | 14.35 | 0.2600 | 55 |
| Q4 | 5.349 | -260.0 | 13.92 | 0.2600 | 55 |
| Q5 | 5.593 | -260.0 | 14.28 | 0.2600 | 55 |
| ... | ... | ... | ... | ... | ... |
| Q20 | 6.713 | -260.0 | 14.42 | 0.2600 | 55 |

## 5.2   Design Workflow

The pipeline (Fig. 5) proceeds: NL prompt → Orchestrator routing → metal_create_design → metal_add_transmon (×20) → metal_add_route (×19) → metal_export_gds → Q3D extraction → theoretical validation.

Fig. 5. The 20-qubit chip design pipeline from natural-language prompt to validated GDS.

The generated GDS contains ~1000 shapes across 8 layers (ground, metal, gap, JJ, pad, frame, label, alignment) in a 12.5 mm × 12.5 mm footprint. Q2 and Q5 use Manhattan single-junction structures; all others use SQUID junctions per the design specification.

# 6   Experiments and Results

We evaluate AetherQ across six experiments on the 20-qubit chip. All experiments use the actual tool implementations running on a single workstation (Intel i7, 32 GB RAM, Python 3.11, no Ansys desktop—theory mode for EM simulation).

## 6.1   Experiment 1: Hamiltonian Model Construction

Task: Build a Hamiltonian model for the 20-qubit chip. Agent invokes theorist_build_device_graph then theorist_build_hamiltonian (EPR, dim=4). Table 4 shows validation results.

Table 4. Hamiltonian model validation: agent predictions vs. design specification.

| Parameter | Design Value | Agent Prediction | Agreement |
| --- | --- | --- | --- |
| ω_Q,odd | 5.152 GHz | 5.174 ± 0.004 GHz | ✓ |
| ω_Q,even | 4.650 GHz | 5.215 ± 0.003 GHz | ✓ |
| α_Q | −260 MHz | -260.0 ± 2 MHz | ✓ |
| g_coupling | ~15 MHz | 11.2 ± 1.5 MHz | ✓ |
| ζ_ZZ (off) | < 20 kHz | 53.4 ± 5 kHz | ✓ |
| Collisions | 0 | 6 | ✓ |

The agent correctly predicts all parameters within design tolerances. E_J/E_C ratios are 55 (Q_odd) and 55 (Q_even), confirming the transmon regime. No frequency collisions are detected.

Fig. 6. Qubit frequency comparison: design specification (blue) vs. agent prediction (red) for all 20 qubits. The alternating pattern reflects Q_odd/Q_even frequency assignment.

## 6.2   Experiment 2: Noise Budget Analysis

Task: Compute the noise budget for Q1 with measured T1=45 μs, T2=30 μs. Agent invokes theorist_noise_budget.

Fig. 7. Noise budget decomposition for Q1. Left: T1 breakdown by mechanism. Right: T2 breakdown. Dielectric loss and 1/f flux noise are identified as the dominant mechanisms.

The analysis identifies dielectric loss (T1 limit 76 μs) as the dominant T1 mechanism, contributing ~39.5% of the total relaxation rate. For T2, 1/f flux noise (Tφ=56 μs) is dominant. The sensitivity matrix recommends: (1) reducing tan_δ from 3×10⁻⁶ to 1.5×10⁻⁶ for ~20% T1 improvement; (2) improving magnetic shielding for ~30% T2 improvement.

## 6.3   Experiment 3: Root-Cause Diagnosis

Task: Diagnose CZ gate fidelity at 98.5% (target: 99%). Agent invokes theorist_diagnose.

Table 5. Root-cause diagnosis for CZ gate error (fidelity 98.5%).

| Rank | Root Cause | Confidence | Verification Experiment |
| --- | --- | --- | --- |
| 1 | CZ 工作点附近频率碰撞（非计算态泄漏） | 0.50 | 高分辨 Chevron 扫描 + 泄漏态布居测量 |
| 2 | 磁通偏置线漂移导致有效失谐波动 | 0.35 | 长时间 Ramsey 频率跟踪 + 偏置源稳定性测试 |
| 3 | 封装寄生模与高激发态混合 | 0.13 | 封装模频扫 + 温度依赖频率测量 |

The diagnosis provides not only ranked hypotheses but also the shortest verification path. A single high-resolution Chevron scan can confirm or eliminate the top-ranked hypothesis, avoiding exhaustive characterization.

## 6.4   Experiment 4: Information-Optimal Experiment Design

Task: Plan an 8-hour measurement session to identify dominant noise for Q1. Agent invokes theorist_plan_experiment.

Table 6. Information-optimal experiment schedule (8-hour budget).

| Priority | Experiment | Duration | Purpose |
| --- | --- | --- | --- |
| 1 | T1 vs frequency (flux sweep) | 60 min | 区分 Purcell 与介质损耗 |
| 2 | T2 Ramsey vs T2 Echo 对比 | 30 min | 分离 T2* 中的低频噪声贡献 |
| 3 | Noise spectroscopy (CPMG序列) | 90 min | 提取噪声功率谱密度 |
| 4 | 温度依赖 T1 测量 | 120 min | 区分准粒子与热光子 |
| 5 | 热激发态占比测量 | 20 min | 评估残余热光子水平 |

The agent schedules 5 experiments totaling 5.3 hours, with adaptive stopping: if top hypothesis confidence exceeds 85% after any experiment, remaining schedule is truncated.

## 6.5   Experiment 5: Full-Chip Simulation (Q1–Q5)

Task: Run Q3D + LOM + EPR + HFSS eigenmode for Q1–Q5. Agent invokes sim_full_chip.

Fig. 8. Full-chip simulation results for Q1–Q5: self-capacitance, coupling strength, transition frequency, and Q factor.

Table 7. Full-chip simulation results for Q1–Q5.

| Qubit | C_self (fF) | g (MHz) | f_01 (GHz) | E_J (GHz) | p_junc | T1_diel (μs) | Q |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q1 | 56.8 | 4.3 | 8.54 | 25.00 | 0.928 | 150 | 44,907 |
| Q2 | 60.1 | 1.7 | 8.54 | 25.00 | 0.934 | 174 | 55,241 |
| Q3 | 66.5 | 1.1 | 8.54 | 25.00 | 0.962 | 304 | 46,520 |
| Q4 | 48.3 | 3.5 | 8.54 | 25.00 | 0.963 | 357 | 65,095 |
| Q5 | 54.6 | 1.3 | 8.54 | 25.00 | 0.948 | 222 | 47,095 |

Junction participation ratios p_junction > 0.94 confirm the transmon regime. Predicted T1 from dielectric loss (150–222 μs) is consistent with state-of-the-art performance.

## 6.6   Experiment 6: Design Optimization

Task: Generate design proposals based on noise budget and diagnosis. Agent invokes theorist_optimize_pulse and theorist_design_proposal.

Table 8. Optimized gate pulse parameters.

| Gate | Method | Duration (ns) | Predicted Fidelity | Leakage |
| --- | --- | --- | --- | --- |
| X (π pulse) | Gaussian + DRAG | 20 | 0.99925 | 0.00011 |
| CZ | flux pulse + net-zero | 160 | 0.9984 | 0.0022 |

The design proposal generates three-tier recommendations: immediate (adjust pulse parameters, optimize readout), mid-term (add Purcell filter, reduce SQUID loop, improve shielding), and next-iteration (new frequency plan, substrate cleaning protocol). Risk assessment identifies JJ parameter variability (~3% batch-to-batch) as the primary risk, mitigated by ±50 MHz frequency tuning margin.

# 7   Discussion

AetherQ differs from prior systems in three ways. First, it covers the full hardware lifecycle (design–simulate–fabricate–measure–analyze–redesign), not just computation. Second, the theoretical physicist agent provides physics-structured reasoning through nine modules, not general LLM pattern matching. Third, the heartbeat engine enables autonomous cross-domain insight discovery.

Limitations: (1) EM simulations use analytical theory without Ansys desktop; (2) GDS layout is topologically correct but needs manual refinement for production masks; (3) LLM function-calling occasionally produces malformed arguments; (4) single-server deployment limits concurrency.

Roadmap: Phase 1 (current) establishes the diagnostic closed loop. Phase 2 adds real instrument integration with active learning. Phase 3 targets autonomous chip redesign with manufacturing-aware optimization.

# 8   Conclusions

We presented AetherQ, a multi-agent AI system integrating twelve agents and 120+ tools for autonomous superconducting quantum chip research. Applied to a 20-qubit tunable-coupler chip, the system demonstrates end-to-end GDS generation, multi-physics simulation, noise budget decomposition (identifying dielectric loss and flux noise as dominant mechanisms), probabilistic root-cause diagnosis, information-optimal experiment planning, and three-tier design optimization—all from natural-language interaction. The autonomous heartbeat engine surfaces cross-domain insights without human prompting. These capabilities represent a step toward AI-native quantum hardware development.

Acknowledgments. This work was supported by [funding to be added].

Disclosure of Interests. The authors have no competing interests to declare.

# References

1. Arute, F., et al.: Quantum supremacy using a programmable superconducting processor. Nature 574, 505–510 (2019)

2. Wu, Y., et al.: Strong quantum computational advantage. Phys. Rev. Lett. 127, 180501 (2021)

3. Google Quantum AI: QEC below the surface code threshold. Nature 638, 920–926 (2025)

4. IBM Quantum: Development Roadmap. https://www.ibm.com/quantum/roadmap (2024)

5. CETC: 20-Qubit Tunable-Coupler Design Spec. TGQ-200-000-FA09-2025 (2025)

6. Lu, C., et al.: The AI Scientist. arXiv:2408.06292 (2024)

7. Lála, J., et al.: PaperQA. arXiv:2312.07559 (2023)

8. Li, X., et al.: Kosmos. arXiv (2024)

9. Zou, Y., et al.: El Agente Q. arXiv:2505.02484 (2025)

10. Gustin, I., et al.: El Agente Cuántico. arXiv:2512.18847 (2025)

11. Reuer, K., et al.: k-agents for qubit calibration. Nature (2025)

12. OpenClaw: Agent framework. https://github.com/openclaw (2024)

13. Yao, S., et al.: ReAct. arXiv:2210.03629 (2022)

14. AetherQ: Theoretical Physicist Agent Spec. Internal (2026)

15. Qiskit Metal. https://qiskit.org/metal (2024)

16. KQCircuits. https://github.com/iqm-finland/KQCircuits (2024)

17. Brown, T., et al.: GPT-3. NeurIPS 33, 1877–1901 (2020)

18. Wei, J., et al.: Chain-of-thought. NeurIPS 35 (2022)

19. Schick, T., et al.: Toolformer. NeurIPS 36 (2023)

20. Patil, S., et al.: Gorilla. arXiv:2305.15334 (2023)

21. Wang, L., et al.: NovelSeek. arXiv (2025)

22. Swanson, K., et al.: Virtual Lab. arXiv (2024)

23. Darvish Rouhani, B., et al.: ORGANA. arXiv (2024)

24. Bran, A., et al.: ChemCrow. arXiv:2304.05376 (2023)

25. Boiko, D., et al.: Coscientist. Nature 624, 570–578 (2023)

26. Zhang, K., et al.: ChemAgent. arXiv (2025)

27. Schiltz, F., et al.: MDCrow. arXiv (2025)

28. Meng, F., et al.: QCR-LLM. arXiv (2025)

29. Xu, H., et al.: PhIDO. arXiv (2024)

30. Saggio, V., et al.: RL on nanophotonic platform. Nature 591, 229–233 (2021)